mn07.gif

Power supply protection - Sprague .003 microfarads each.  Two in

series to ground on each high voltage bushing.  Safety gap.  Chokes

are 2" x 1" x 1/2" powdered ferrite cores with 16 turns of #12

stranded copper hookup wire each.

High voltage lines to tank are #14 neon sign wire.